# Procedure for centering RUDDERs on the Knysna 500 and Knysna 500 se

# Hydraulic oil reservoir

### Ensure the hydraulic reservoir at the helm pump has sufficient hydraulic fluid

### Recommend hydraulic oil is Shell Tellus S2M industrial hydraulic fluid.

# Hydraulic rams and rod ends

Ensure that the rod ends are secured on the ram of the hydraulic cylinder and that the retaining pin is in position on the tiller arm.

# CENTRING THE TWO RUDDERS

## Turn the helm wheel hard to port,

## Go to the bypass valve on the port side and open this valve.

## Return to the helm and continue turning to port until the wheel stops turning. When it stops return to the port side bypass valve and close it.

## Go to the starboard bypass valve and open it.

## Return to the helm and turn the wheel to port until the wheel stops turning.

## Return to the starboard bypass valve and close it.

## The rudders are now equalised.

# Emergency tiller installation and operation

## Go to the starboard rudder

## Remove the grey cap on the top of the rudder post

## Locate the emergency tiller into the rudder post

## Open the bypass valve on the starboard side

## The emergency tiller will now move the starboard rudder

# Please note: only the starboard rudder is used in emergencys.